16-bit ripple carry adder (RCA)

module half_adder(output S, C, input x, y);

xor #(20)(S, x, y);

and #(10)(C, x, y);

endmodule

module full_adder(output sum, carry, input x, y, z);

wire S1, C1, C2;

half_adder ha1(S1, C1, x, y);

half_adder ha2(sum, C2, S1, z);

or #(15) G1(carry, C2, C1);

endmodule

module ripple_carry_4_bit_adder(output [3:0]s, output cout,  input[3:0] X, Y, input Z);

wire C1, C2, C3;

full_adder fa1(s[0], C1, X[0], Y[0], Z);

full_adder fa2(s[1], C2, X[1], Y[1], C1);

full_adder fa3(s[2], C3, X[2], Y[2], C2);

full_adder fa4(s[3], cout, X[3], Y[3], C3);

endmodule

module ripple_carry_16_bit_adder(output[15:0]aaa, output cout, input[15:0] A, B, input cin);

wire C1, C2, C3;

ripple_carry_4_bit_adder rca1(aaa[3:0], C1, A[3:0], B[3:0], cin);

ripple_carry_4_bit_adder rca2(aaa[7:4], C2, A[7:4], B[7:4], C1);

ripple_carry_4_bit_adder rca3(aaa[11:8], C3, A[11:8], B[11:8], C2);

ripple_carry_4_bit_adder rca4(aaa[15:12], cout, A[15:12], B[15:12], C3);

endmodule

module testbench_ripple_carry_16_bit_adder;

reg[15:0]A, B;

reg Cin;

wire[15:0]Sum;

wire Cout;

ripple_carry_16_bit_adder rca16bit(Sum, Cout, A, B, Cin);

initial begin

A = 0; B = 0; Cin = 0;

#200 A = 16'b0110100101010101; B = 16'b0101010101010101; Cin = 1'b0;

#100 A = 16'b0000001000010000; B = 16'b0000000000100010; Cin = 1'b1;

#100 A = 16'b0101000011110000; B = 16'b0000011111110000; Cin = 1'b0;

#100 A = 16'b0101000000011100; B = 16'b0000000000111111; Cin = 1'b1;

end

initial #800 $finish;

initial $dumpvars;

endmodule

16-bit carry lookahead adder (CLA)

module carry_look_ahead_16_bit(input[15:0] a, b, input cin, output [15:0] sum, output cout);

wire c1,c2,c3;

carry_look_ahead_4_bit cla1(a[3:0], b[3:0], cin, sum[3:0], c1);

carry_look_ahead_4_bit cla2(a[7:4], b[7:4], c1, sum[7:4], c2);

carry_look_ahead_4_bit cla3(a[11:8], b[11:8], c2, sum[11:8], c3);

carry_look_ahead_4_bit cla4(a[15:12], b[15:12], c3, sum[15:12], cout);

endmodule

module carry_look_ahead_4_bit(a, b, cin, sum, cout);

input[3:0] a, b;

input cin;

output[3:0] sum;

output cout;

wire [3:0] P, G, C;

assign #10 P = a ^ b;

assign #10 G = a & b;

assign #10 C[0]= cin;

assign #10 C[1]= G[0] | (P[0] & C[0]);

assign #10 C[2]= G[1] | (P[1] & G[0]) | P[1] & P[0] & C[0];

assign #10 C[3]= G[2] | (P[2] & G[1]) | P[2] & P[1] & G[0] | P[2] & P[1] & P[0] & C[0];

assign #10 cout= G[3] | (P[3] & G[2]) | P[3] & P[2] & G[1] | P[3] & P[2] & P[1] & G[0] | P[3] & P[2] & P[1] & P[0] & C[0];

assign #10 sum = P ^ C;

endmodule

module testbench_carry_look_ahead_16_bit;

reg [15:0] a,b;

reg cin;

wire [15:0] sum;

wire cout;

carry_look_ahead_16_bit cla16bit(a, b,cin,sum,cout);

initial

begin

a=0; b=0; cin=0;

#100 a= 16'b0000000111110000; b=16'b000101100001100; cin=1'b0;

#100 a= 16'b1100011010000101; b=16'b000000010000001; cin=1'b1;

#100 a= 16'b0000001111110000; b=16'b011100000000000; cin=1'b0;

#100 a= 16'b0000000000010000; b=16'b000010000000001; cin=1'b1;

end

initial #500 $finish;

initial $dumpvars;

endmodule

A : 前瞻式進位加法器 (CLA) 較 漣波進位加法器 (RCA)快。

C :  RCA, 因為他所用用到的晶片面積複雜度比較低。

D : 漣波加法器 : 速行運算時速度慢，但運用的邏輯閘數比較少，因此布局簡      單較容易設計。

前瞻加法器 : 執行運算起起來速度快，但所需要用用到的邏輯閘數比較多。如果運算的位元數較多的話，電路會變得十分複雜。